# Modeling, Simulation, and Visualization of a Robotic Manipulator

KUKA KR 6 R900 sixx (AGILUS) 6-DOF

Algebra for the equations is in BLISSjanga_Handwritten. Python code that produced the figures and numerical results is in the Jupyter notebook BLISSjanga_FinalProject.ipynb.

# Overview

I decided to pick the KUKA KR 6 R900 sixx from the KR AGILUS family. This robot is a 6-DOF industrial arm with all revolute joints and a true spherical wrist (axes 4, 5 and 6 all meet at one point). The spherical wrists give closed-form inverse kinematics, so Part 6 becomes an algebra problem instead of a numerical one. The PUMA-like shoulder offset (a1=25 mm) adds shoulder front and shoulder back branches, which made the IK slightly more interesting.

The math in this report uses the DH convention used in class. The derived DH table was cross-checked against the DH figure in Turgut & Kaleli (2022, MDPI Machines).

All code runs in Python 3 inside the conda env rvc (numpy, matplotlib, spatialmath-python, urchin). Running the code regenerates every figure and animation here.

# Problem 1 - Robot Identification: KR 6 R900 sixx

I chose the KUKA KR 6 R900 sixx from the KR AGILUS family after seeing KUKA arms building Mercedes cars during a visit to a manufacturing plant in Germany. Watching a factory of these arms weld, handle, and position sheet-metal panels fully coordinated together, quickly and even in a confined space made the precision and compactness of the AGILUS design very interesting. From a project standpoint, the KR 6 R900 sixx is interesting as well because of its true 6R arm with a spherical wrist, which is nice because it gives closed-form inverse kinematics, and because the ROS-Industrial community maintains a well-documented URDF that I can cross-check my DH model against. Its compact size (≈52 kg, 0.9 m reach).

KR 6 R900 sixx  Specs.

Source: KUKA OI BA KR AGILUS sixx V12 (2015).

| Manufacturer | KUKA Roboter GmbH (Augsburg, Germany) |
| --- | --- |
| Family / model | KR AGILUS sixx / KR 6 R900 sixx |
| Joints | 6 revolute, in-line (spherical) wrist |
| Reach | 0.901 m to wrist centre (0.981 m to flange) |
| Payload | 6 kg max, 3 kg rated |
| Arm mass | ≈52 kg |
| Repeatability | ±0.03 mm (ISO 9283) |
| Working volume | 2.85 m​3 |
| Protection | IP54 (WP/HM variants: IP67) |
| Actuation | brushless AC servo motors with cycloidal (RV-type) reducers on the main axes A1-A3; A4 uses a smaller gear stage, while A5 and A6 are driven via toothed belts that decouple motor placement from the wrist joint axes, reducing distal mass (KUKA OI §3.2, §9.4) |
| Sensing | per-joint resolvers; motor brakes on every axis |
| Controller | KR C4 compact |

Kinematically, the KR 6 R900 sixx is an anthropomorphic 6R manipulator. Joint A1 allows the robot to have vertical base rotation. Joints A2 (shoulder pitch) and A3 (elbow pitch) form a planar two-link arm in the vertical plane, that have a small 25 mm shoulder offset a₁ which gives the arm a PUMA-like geometry. This offset shifts the shoulder singularity locus from just a line to a thin cylinder around the A1 axis. Joints A4 (wrist roll), A5 (wrist bend), and A6 (tool roll) create aspherical wrist whose three axes all intersect at a single point, the wrist center, that is located 420 mm from the elbow along the forearm. The upper arm length is 455 mm (a₂), and the flange sits 80 mm beyond the wrist centre (d₆). Some typical applications for this robot include high-speed small-part handling, machine tending, assembly, and arc welding in compact, safety-fenced cells. Seeing these applications at the Mercedes plant is what made me so interested.

Joint limits (radians):

qmin=[-2.9671, -3.3161, -2.0944, -3.2289, -2.0944, -6.1087]​⊤,qmax=[+2.9671, +0.7854, +2.7227, +3.2289, +2.0944, +6.1087]​⊤,

# Problem 2 - DH Frames and DH Table

## The Rules for assigning frames (Standard DH)

Frames are assigned following the three rules from the handout:

- zi-1 is along the axis of joint i.

- xi lies along the common normal from zi-1 to zi (pointing from zi-1 toward zi).

- yi completes a right-handed frame.

Using these conventions, the link parameters (ai,αi,di,θi) are defined as:

ai=distance from zi-1 to zi along xi,αi=angle from zi-1 to zi about xi,di=distance from xi-1 to xi along zi-1,θi=angle from xi-1 to xi about zi-1.

For a revolute joint the variable is θi ( θi=qi plus any offset); ai,αi,di are constants.

Below on figure 2, it shows the coordinate frames I assigned to the KR 6 R900 sixx by applying the three rules given above to the robot's physical geometry. The assignment applied joint by joint were:

- Frame 0 (base): z₀ points upward along the A1 rotation axis. x₀ is chosen pointing forward (toward the arm's default reach direction). y₀ completes the right-handed frame but is not necessary to draw.

- Frame 1: z₁ points along the A2 axis (horizontal, perpendicular to z₀). x₁ lies along the common normal from z₀ to z₁, offset radially by a₁ = 25 mm(the shoulder offset) which is kept in the same direction for simplicity. The distance from x₀ to x₁ measured along z₀ is d₁ = 400 mm which is the shoulder height. The twist from z₀ to z₁ about x₁ is α₁ = −π/2.

- Frame 2: z₂ is parallel to z₁ (both A2 and A3 are shoulder/elbow pitch axes and are in the same plane), so α₂ = 0. x₂ is along the common normal from z₁ to z₂, spanning the upper arm length a₂ = 455 mm. There is no offset along z₁, so d₂ = 0.

- Frame 3: z₃ points along the A4 axis, which is perpendicular to z₂ (this is the forearm rotates about a different plane than the elbow pitch). x₃ lies along the common normal from z₂ to z₃, covering the 35 mm elbow offset a₃. The twist from z₂ to z₃ about x₃ is α₃ = −π/2. There is a θ offset of −π/2 which is needed so that q₃ = 0 aligns with the URDF software zero (arm extended horizontally) rather than with x₂.

- Frames 4 and 5 (makes a spherical wrist): z₃, z₄, z₅ all intersect at the wrist centre. z₄ is along the A5 axis, perpendicular to z₃, giving α₄ = +π/2. The distance from x₃ to x₄ along z₃ is the forearm length d₄ = 420 mm. z₅ which is along the A6 axis, perpendicular to z₄, giving α₅ = −π/2.

- Frame 6 (flange): z₆ is along the tool axis. a₆ = 0 and α₆ = 0 (there is no twist or offset beyond the wrist). d₆ = 80 mm is the distance from the wrist centre to the flange face.

These assignments make the six rows of the DH table below. The θ-offsets for joints 3, 4, and 6 (−π/2, −π, −π respectively) align the DH software zero with the ROS-Industrial URDF convention where q = 0 corresponds to the arm fully extended horizontally.

Figure 2-3: DH coordinate frame assignments for the KR 6 R900 sixx at q = 0. (Top) 3D perspective which shows the frame origins at each joint. (Bottom) Side view with dimensions in mm from the KUKA datasheet. Each frame shows xᵢ and zᵢ in red.

## The KR 6 Standard-DH table

DH parameters for the KR 6 R900 sixx. Lengths in m, angles in rad. The θi column is the joint variable qi. q=0 lines up with the ROS-Industrial URDF software-zero (arm fully extended horizontally).

| i | ai [m] | αi [rad] | di [m] | θi [rad] |
| --- | --- | --- | --- | --- |
| 1 | 0.025 | -π/2 | 0.400 | θ₁ |
| 2 | 0.455 | 0 | 0.000 | θ₂ |
| 3 | 0.035 | -π/2 | 0.000 | θ₃ |
| 4 | 0.000 | +π/2 | 0.420 | θ₄ |
| 5 | 0.000 | -π/2 | 0.000 | θ₅ |
| 6 | 0.000 | 0 | 0.080 | θ₆ |

Physical meaning of the non-zero parameters:

- a1=25 mm: shoulder offset (PUMA style).

- d1=400 mm: shoulder height.

- a2=455 mm: upper arm length.

- a3=35 mm: elbow offset between A3 and A4 axes.

- d4=420 mm: forearm length.

- d6=80 mm: wrist-centre to flange.

## The Standard DH link transform

The transform from frame i-1 to frame i is the product of four elementary transforms:

Tii-1=Rz(θi) Tz(di) Tx(ai) Rx(αi).

This was derived in class by expanding the four elementary matrices and multiplying them out. The results are:

Tii-1(ai,αi,di,θi) = cθi-sθi cαisθi sαiai cθisθicθi cαi-cθi sαiai sθi0sαicαidi0001.

I’ll check by substituting q=0 into and chaining through puts the wrist centre at (0.900, 0, 0.435) m. Which is a radial distance from the A1 axis =0.900 m. KUKA publishes 0.901 m (difference from 4-decimal URDF rounding).

To validate DH frames 0 through 7 computed by the FK code at q = 0 (this is the URDF software zero). Each frame shows xᵢ, yᵢ, zᵢ in the base frame. This plot confirms that the DH table above produces frames that are consistent with the physical joint axes shown in Figure 2-3: each zᵢ lines up with joint i + 1, and the x-axes walk along the common normals between consecutive z-axes.

# Problem 3 - Forward Kinematics

## Chain product

The forward kinematics is the product of six link transforms from the DH table

T60(q) = T10(q1) T21(q2) T32(q3) T43(q4) T54(q5) T65(q6).

## Numerical evaluation at the ready pose

The general transformation matrix T⁰₆ was derived by multiplying all six link transforms from the DH table, the derivation for this is worked out in BLISSjanga_Handwritten.pdf. To also check and verify the result numerically, I evaluate at the ready pose (picked to keep q5≠0 so the wrist is not singular, and q3>0 so that the elbow is bent):

qready = [0, -π3, π3, 0, π4, 0]​⊤.

At q1=0:

T10(q1=0)=1000.02500100-100.4000001

The other five link matrices T21,…,T65 at qready are printed by the problem3.py cell on the Jupyter notebook. Multiplying all six link matrices in order produces the pose:

T60​(qready)=-0.707100.70710.72910-1000.707100.70710.88560001.

The flange position p6=(0.7291, 0, 0.8856) m, is the tool pointing forward and up at a 45∘ angle. This was cross checked with the ROS-Industrial URDF in RViz at the same joint values.

#### Intermediate link poses Ti0for qready:

T20=0.50000.866000.252500100.8660-0.500000.79400001,T30=0010.25250-1001000.82900001,T40=0100.672500-10-1000.82900001,T50=0.707100.70710.67250100-0.707100.70710.82900001.

These are the matrices printed by problem3.py in the notebook which are rounded to 4 decimals here for the write-up. Some things that help confirm the angles are frame 4 and 5 share the same origin (0.6725, 0, 0.829), which makes sense. Frames 4, 5, 6 meet at the wrist centre.

The wrist centre origin p4=p5=(0.6725,0,0.829) is good to see for Part 6 where it is what the inverse position IK has to target.

Stick figures from kr6_frames at q=0 (left, URDF software zero) and qready (right).

# Problem 4 - Stick Figure + Joint-Space Quintic Trajectories

## The quintic polynomial trajectory

Let

q(t) = a0+a1t+a2t2+a3t3+a4t4+a5t5.

Six unknowns, six conditions (position, velocity, acceleration at t0 and tf) give a 6×6 linear system (in BLISSjanga_Handwritten.pdf). With q0=q0=qf=qf=0 and t0=0, tf=T. We can set the velocity and acceleration to zero at both endpoints which makes the arm start and arrive at rest with no jerk. Solving the system gives:

a0=q0, a1=a2=0, a3=10 ΔqT3, a4=-15 ΔqT4, a5=6 ΔqT5, Δq=qf-q0,

trajectories.quintic  in the notebook solves the 6×6 system numerically.

#### Numerical example - joint 1, qf=0.8 rad.

With q0=0, qf=0.8, T=2 s, gives a3=1.000, a4=-0.750, a5=0.150. So q1(t)=t3-0.75 t4+0.15 t5 on [0,2]. the quintic profile figure plots this polynomial and its first two derivatives below.

Joint 1 quintic profile q(t),q(t),q(t). Zero boundary velocity and acceleration give a nice smooth start and smooth stop.

## Stick figure and joint-at-a-time motion

The stick figures at q = 0 and q_ready (Problem 3) shows that the FK gves the expected geometry. The joint at a time sweeps below provides additional verification of each joint’s independent effect on the arm. The stick figure is drawn by reading origins {p0,p1,…,p6} from kr6_frames(q) and connecting consecutive origins with line segments. This was applied in Part 4 GIF

q(t) = q₀ + (10Δq/T³)t³ − (15Δq/T⁴)t⁴ + (6Δq/T⁵)t⁵ to one joint at a time with

Δq=[0.8, 0.6, 0.8, 0.6, 0.8, 0.6] rad,

moving out and back .

This is each joint swept individually from the ready pose. The grey ghost is the ready pose and colored figure is the ready pose with only joint j shifted by Δqj.

#### Animation:

# Problem 5 - Jacobian and Singular Configurations

## Direct-method geometric Jacobian

For a configuration q, the flange velocity [v;ω] expressed in the base frame is always related to q by the 6×6 geometric Jacobian (square because the KR 6 has 6 joints):

v0,6 0ω0,6 0=J(q) q.

Every joint is revolute, so every column uses the same formula from the JacobianGuide.pdf provided in class:

Ji(q) = zi-1 0×(t0,6 0-t0,i-1 0)zi-1 0, i=1,…,6.

zi-1 0 is the third column of Ri-1 0, and t0,i-1 0 is the origin of frame i-1 in base coordinates. Both come directly from {Ti-10} returned by kr6_frames.

#### Derivation

Joint i is spinning at rate qi about zi-1 0 which contributes ωi=qizi-1 0 to the flange’s angular velocity. Rigid-body velocity then gives vi=ωi×(t0,6 0-t0,i-1 0)=qi zi-1 0×(t0,6 0-t0,i-1 0) for the upper block.

## J(qready)

Plugging qready into the equation, it gives

J(qready)=00.48560.09160-0.056600.729100-0.0566000-0.7041-0.476600.05660000100.70710110-10100000.7071.

The detJ(qready)=-0.0749.

## Singular configurations

J loses rank when the end-effector can no longer move instantaneously in some direction of a task space. For the 6×6 KR 6 Jacobian this happens when detJ=0, which for a 6R arm with spherical wrist happens in three distinct geometric families:

Shoulder singularity

This happens when the wrist centre lies on (or very close to) the A1 axis. Because a1=0.025 m is non-zero, the singular set is a cylinder of radius a1 around the A1 axis instead of the axis itself.

- Numerical check: at q = [0, −1.760, 0.417, 0, π/4, 0] the wrist centre sits at r_wc ≈ 0 m (directly above A1); det J ≈ 10⁻¹⁰ ≈ 0.

Elbow singularity

This is when the shoulder, elbow and wrist centre line up (the arm is fully extended). In terms of the bend βe in IK, this is βe→0.

- Numerical check: at q = [0, 0, 0.0831, 0, π/4, 0] the arm is fully extended (q₃ = β = atan2(a₃, d₄), so β_e = 0); det J ≈ 10⁻¹⁷ ≈ 0.

Wrist singularity.

Happens when axes A4 and A6 become collinear (q5→0). It makes the wrist lose one orientation DOF and the Jacobian submatrix for the last three columns drops rank.

The Yoshikawa manipulability measure

w(q) = |det​(J(q)J(q)​⊤)|

Which is a scalar that goes away at each of these singular configurations.

#### Wrist-singularity sweep.

I decided to sow the wrist singularity because it is the one a trajectory planner is most likely to encounter: a tool approaching a surface normally runs q5 through zero. Starting from qready I swept q5 linearly from +π/4 down to 0, holding all other joints fixed, and recorded σmin(J), w(q) and |detJ| at each step. The singularity sweep figure shows all three signals drop as q5→0, a standard wrist-singularity signature.

σmin(J) (top) and w(q) + |detJ| (bottom) along a wrist-straightening sweep q5:π4→0. All three signals drop to zero at q5=0 (which is wrist singularity).

## Velocity-arrow visualization

Given any q, the flange twist comes from the Jacobian definition Equation.

Example:

q=(0.6, 0.4, -0.3, 0.5, 0.3, 0)​⊤ rad/s at the ready pose,

v=Jv(qready) q=(0.150, 0.409, -0.122)​⊤ m/s,  |v|=0.452 m/s,

ω=Jω(qready) q=(0.500, -0.200, 0.600)​⊤ rad/s,  |ω|=0.806 rad/s.

the velocity-arrows figure draws these as arrows on the stick figure.

Stick figure at qready with the linear velocity arrow v (magenta) and the scaled angular velocity arrow ω (cyan) on the flange.

The animation below draws the linear velocity arrow v=Jv(q)q (magenta) and angular velocity arrow ω=Jω(q)q (cyan) on the flange as the arm moves along a short joint-space quintic.

#### Animation:

# Problem 6 - Inverse Position and Velocity Kinematics

KR 6 has a spherical wrist. The IK is closed-form. This derivation splits into four steps:

- Wrist centre pwc. Point where A4, A5, A6 all intersect. It lies a distance d6 behind the flange along the tool axis. Just one vector subtraction.

- q1. Azimuth of pwc in the base frame. My a1 is a radial offset along x1, so the standard PUMA “lift” term goes away and q1 is a single atan2 call.

- q2,q3. Planar 2R triangle with vertices at shoulder, elbow, wrist centre. The elbow offset a3 and the forearm d4 fold into a single virtual forearm of length L=a32+d42.

- q4,q5,q6. ZYZ Euler decomposition of R3 6=R0 3 ⊤Rd.

Inverse velocity IK follows from the square Jacobian: q=J(q)-1[vd;ωd]. No damping, no pseudoinverse.

## Step 1 - The wrist centre

Let the desired flange pose be T​d=Rdpd0​⊤1∈SE(3). The wrist centre pwc is on the tool axis, offset by d6 behind the flange origin:

pwc = pd-d6 Rd ez,

where ez=(0,0,1)​⊤, so Rdez is the third column of Rd, where pwc is independent of q4,q5,q6.

## Step 2 - q1 from the wrist centre’s azimuth

a1=25 mm in the DH table above is a radial offset along x1, so after the q1 rotation the shoulder sits on the arm’s radial line from the A1 axis. The wrist centre which means it lies in the same half-plane at azimuth q1. With pwc=(xw,yw,zw):

q1=atan2(yw, xw).

A second branch q1=atan2(yw,xw)+π, this means shoulder rotated 180∘, arm pointing backward is also reachable. The shoulder-back branch.

## Step 3 - q2,q3 from a planar 2R triangle

With q1 fixed, the problem collapses to the arm plane (the plane containing the upper arm and the A2 axis). Using the arm-plane coordinates (r,z) with r the radial distance from A1 and z the world z:

A2 (shoulder) at (a1, d1)=(0.025, 0.400),wrist centre at (±rxy, zw), rxy=xw2+yw2,

with the sign of rxy set by the shoulder branch where + is for front, - is for back.

#### The virtual-forearm trick

From A3 to the wrist centre is a 35-mm perpendicular elbow offset followed by a 420-mm forearm. Combining them into a single virtual forearm of length

L=a32+d42≈0.4215 m,  β=atan2(a3,d4)≈0.0832 rad,

which turns the three-segment chain of upper arm,  elbow offset, and forearm into a 2R planar linkage with legs a2 and L offset by β.

#### 2R triangle.

With sx=±rxy-a1, sz=zw-d1, dp=sx2+sz2, and βe being the “bend” between upper arm and virtual forearm, law of cosines gives

cosβe = dp2-a22-L22 a2 L,  βe>0 is elbow up, βe<0 is elbow down.

This means joint variable q3 is the bend plus the constant elbow-offset angle β (this is at q3=0 the virtual forearm is already tilted above the upper arm by β, and not aligned with it):

q3 = βe+β.

For q2, let ψ=atan2(sz,sx) be the angle of the shoulder-to-wrist line. The law of sines at the shoulder vertex gives

ϕ=atan2​(Lsinβe, a2+Lcosβe),

and flipping the sign from α1=-π/2 in the DH table gives

q2=-(ψ+ϕ) (elbow up), q2=-(ψ-ϕ) (elbow down).

## (from the class derivation.)

## Step 4 - q4,q5,q6 from ZYZ Euler

With q1,q2,q3 known, FK gives R0 3 and the wrist must produce R3 6=R0 3 ⊤Rd. Substituting the α and θ-offsets from the DH table above into the DH product and also simplifying using the identity R_x(π/2) R_z(q₅) R_x(−π/2) = R_y(−q₅):

R3 6 = Rz(q4-π) Ry(-q5) Rz(q6-π).

This is a ZYZ Euler decomposition with shifted first and third angles. Taking out Euler angles from R3 6 with the standard formulas, then using the sign ambiguity (α,β,γ)∼(α+π,-β,γ+π) to pick q5≥0 (this cancels the two -π offsets from the DH table above) gives the final joint-angle assignments:

q4=atan2​([R3 6]23, [R3 6]13), q5=atan2​([R3 6]132+[R3 6]232, [R3 6]33), q6=atan2​([R3 6]32, -[R3 6]31).

## (following the ZYZ extraction method from class.)

## Validation

Equations wrist-centre, q1 azimuth, q3 elbow-bend, q2 shoulder-lift, and ZYZ Euler are what we need to use algebra to map from T​d to q, given a choice of elbow-up/down and shoulder-front/back. A way to do a quick check by round-tripping is to pick 500 random q∼uniform([qmin,qmax]) with q5>0 so the wrist is not singular, compute T​d=T60(q), run IK, then FK applied to the IK solution should reproduce T​d. Code (problem6.py::round_trip) reports

max∥pFK-pd∥≈6×10-16 m,max∥RFK-Rd∥F≈2×10-15.

Both residuals are roundoff-limited (double-precision).

Target pose generated by FK of qtrue (left) vs. IK reconstruction returned by the closed form (right). The two stick figures and flange frames match.

## Branches

For a reachable target there are up to four position-level branches: (elbow-up or elbow-down) × (shoulder-front or shoulder-back). In the code these are the elbow_up and shoulder_front flags of ik_position. Another ambiguity (q5>0 vs. q5<0) comes from the wrist, in this case my sign convention from the ZYZ Euler equation always uses the positive branch.

Four position-level IK branches for one flange target: elbow up/down × shoulder front/back. Each q satisfies T60(q)=T​d where they differ in how the arm is folded.

## Inverse velocity kinematics

The 6×6 Jacobian J(q) in from the Jacobian definition equation is square. It is invertible everywhere except at the three named singularities in problem 5. Inverse velocity IK is then a nice direct solve:

q = J(q)-1vdωd.

There is no damping or pseudoinverse. Next we will go to problem 7 where we will put the inverse-velocity equation on a moving reference and compare against ddtq(t) to check for independent consistency.

# Problem 7 - Task-Space Trajectory with Position and Velocity IK

## Cartesian quintic trajectory

Two flange poses as small offsets from the original ready position were picked so the path stays inside a folded-elbow region and away from the three singularities we talked about earlier:

pstart=pready+(-0.15, +0.05, +0.05)​⊤m, pend=pready+(-0.05, -0.08, -0.05)​⊤m.

Orientation is fixed at Rfixed=Rready throughout, so ωd(t)≡0. Each Cartesian axis ξ∈{x,y,z} gets its own quintic with T=2 s:

ξ(t)=ξ0+Δξ (10s3-15s4+6s5),  ξ(t)=ΔξT(30s2-60s3+30s4), s=t/T.

Stacking them gives the task-space reference pd(t), vd(t). The peak task-space speed along this path is ∥vd∥max≈0.180 m/s.

## Inverse position kinematics - q(t)

At each of the N=80 uniform time samples tk  calling the closed-form IK (wrist-centre equation)-(ZYZ Euler equation) against T​d(tk)=(Rfixed, pd(tk)), while also keeping the same branch (elbow-up and shoulder-front) at every step. This makes each call one pass through the equations, with no iteration.

Inverse Position kinematics: q(t) for the six joints over the 2 second Cartesian quintic, which were all produced by the closed-form solver.

## Inverse velocity kinematics - q(t)

At each q(tk), we will compute q(tk) directly from inverse-velocity equation:

q(tk) = J(q(tk))-1 vd(tk)0.

Logging  σmin(J) at every sample where the minimum over the whole path is ≈0.1, so the inverse is numerically safe.

#### Consistency check

Compute the central-difference derivative of the position-IK trajectory,

qj(tk)=(qj(tk+1)-qj(tk-1))/(2Δt), and compare to the velocity-IK output qj(tk). So since the finite-difference estimate is independent of (inverse-velocity Eq.) but describes the same trajectory, they should match:

maxj,k|qj(tk)-qj(tk)|≈3×10-4 rad/s, RMS≈8×10-5 rad/s,

against ∥q∥∞≈0.48 rad/s. Roughly 3-4 significant figures of agreement. Position-IK and velocity-IK are consistent.

Inverse velocity kinematics: q(t) computed directly from J-1vtask (solid line) overlaid with the finite-difference derivative of q(t) (dashed line). The two curves are indistinguishable on every joint.

The start stick figure is blue, end stick figure is orange, and the flange path is red which was produced by the closed-form position IK.

#### Animation:

#### What happens near a singularity

If a path crossed a wrist singularity (q5→0) or an elbow singularity (βe→0), J-1 would blow up. A good workaround is to damp the least-squares pseudoinverse J​⊤(JJ​⊤+λ2I)-1 inside a small neighborhood or inserting a waypoint that avoids the area entirely. We didn’t have to use either of the workarounds for this path.

# Problem 8 - URDF Creation and Animation

## Goal is to create a URDF whose link frames are the DH frames

So, the issue of URDF is that it is a very broad format, and it does not tailor towards a specific kinematic convention. This causes different URDFs of the same physical robot having their link frames in very different places, depending on who created the file. This causes things like trajectories computed in my DH code to not work, in general. Which causes the transfer to an URDF of the same robot to not work without changing each link.

What is needed to fix this issue is a URDF whose link frames are the same to the frames my DH code uses. This would make it so a q vector that drives the stick figure to also drive the URDF render in the same way without needing additional changes. Doing this would make the wrist centre in this URDF sit at the same world coordinates that Problem 6 targeted, and the task-space trajectory from Problem 7 replays through the URDF without any need for re-mapping.

## Why a single URDF revolute joint is not enough

The URDF says that every revolute joint has a fixed origin transform from parent link to joint axis, which then causes the joint variable to rotate about that axis:

ᵖᵃʳᵉⁿᵗT_child = Ξᵢ · R_z(qᵢ)

The full Standard-DH link transform factors as

Tii-1(qi) = Rz(θoff,i) Tz(di) Rz(qi) Tx(ai) Rx(αi) = Rz(θoff,i) Tz(di)⏟prei Rz(qi) Tx(ai) Rx(αi)⏟posti,

which is because Rz and Tz move along z. A single URDF revolute joint can realise prei⋅Rz(qi), but it cannot also do the posti factor that comes after Rz(qi). If I tried to compress Tii-1 into just one revolute joint, URDF link i would land at the pre-rotation point, and not at DH frame i, which causes the two descriptions to diverge by up to ​ai at every joint.

## The fix is to split each DH link into a revolute joint and a fixed joint

I followed what we discussed and added an auxiliary pivot_i and a fixed joint FJi after each revolute joint, so this makes the URDF chain for link i to be

link_{i-1} → [revolute joint_i, origin = pre_i, axis = e_z] → pivot_i → [fixed FJ_i, origin = post_i] → link_i

With this structure, the URDF transform from linki-1 to linki is

Ξirev Rz(qi) Ξifix = prei⋅Rz(qi)⋅posti = Tii-1(qi),

which is exactly the DH link product from DH pre/post factoring equation that we are looking for. So if URDF link i is the same SE(3) pose as DH frame i for any q, the pre_i and post_i factors get to be computed directly from the DH table, where URDF writer in problem8.py(write_urdf) numerically evaluates them and serializes as xyz translations and rpy Euler angles.

## Numerical validation across four configurations

Loading urdf/kr6_sdh.urdf with urchin and calling link_fk(cfg=q) returns the URDF’s link origins for any q. Using this we can compare those origins to t0,i 0 from kr6_frames(q) at four test configurations

Maximum position and rotation mismatch between URDF link i and DH frame i over i=0,…,6, woth the residual floor set by the four-decimal rounding of xyz / rpy in the URDF.

| configuration | max position error [mm] | max rotation Frob. error |
| --- | --- | --- |
| qzero=0 | 0.003 | 2.7×10-5 |
| qready=[0,-π3,π3,0,π4,0] | 0.002 | 3.1×10-5 |
| twisted q=[0.5,-0.6,0.7,0.8,0.5,-0.3] | 0.002 | 2.9×10-5 |
| folded q. =[-1.0,-1.2,1.5,0.2,0.8,0.1] | 0.002 | 3.1×10-5 |

The per-configuration max error stays at the 3-micrometre rounding floor even when aggressive off-zero configurations are used, which confirms the URDF link frames are the DH frames and not just close or nearly the same for every reachable q.

## Trajectories transfer directly

Due to the URDF link frames being the DH frames, the outputs I generated in earlier problems transfer directly to the URDF model.

- The stick figure from kr6_frames is drawn by connecting the DH frame origins, and the URDF link origins are the same points, so the two plots would be the same.

- The joint trajectory q(t) saved as figures/problem7_Q.npy from Problem 7 would drive the URDF through the same SE(3) motion as the stick figure at every timestep.

- Waypoint, flange target, or velocity profile built in the DH code would work as a URDF waypoint without additional changes.

Stick figure (left) vs URDF cylinders (right) at qready, drawn from using the same q vector as well as using the same link frames. Cylinder lengths and radii are just visual, so they do not represent the AGILUS’s external sheet-metal geometry.

The URDF-vs-stick animation over this box path shows the DH stick figure (left) and the URDF cylinders (right) at the same q at every frame. They are symmetric at every time step because the link frames are the same.

#### Animation:

# Deliverables

- BLISSjangaFinalProject.pdf / .docx — this report and its Word source

- BLISSjanga_FinalProject.ipynb: Jupyter notebook with every line of code that produced the figures and numerical results, organised by Problem

- BLISSjanga_Handwritten.pdf: iPad derivations (DH frame assignment, link-transform matrix expansion, six individual link transforms, general T⁰₆ symbolic multiplication, quintic coefficient solve)

- code/kuka_dh.py - DH table, link transform (link-transform Eq.), FK chain (FK chain Eq.)

- urdf/kr6_sdh.urdf - hand-authored URDF with revolute + fixed-joint pairs so link frames coincide with DH frames

Every numerical value in this report is produced by running the notebook cells in the rvc conda env.